Overview:

In the Pet Car Alarm project, students build and program a simple feedback and control loop. Feedback and control loops are central to many industrial systems and consumer products.  The system uses three input modules: two temperature sensors and a Hall effect magnetic field sensor. The system has four outputs that include two white LED lights (headlights), one continuous servo motor (controls the window) and also makes use of the TI-Innovator Hub’s built-in speaker (horn honking). The continuous servo requires more power than the calculator can provide. Therefore, an external USB battery is needed.  A program is written to read all three input parameters and logically compare them with critical set-point values to determine when to turn on the alarm. The project is presented in a series of small challenges that build the knowledge and skills required for the final, ore open-ended challenge. The final challenge also relies on the students’ understanding of important biology and Earth science topics that are relevant to optimizing the system the students ultimately design and refine.

Note: This project assumes the students have a working knowledge of the programming concepts in the Digital Mood Ring project. Please refer to that project as a review of concepts. The materials for the project can be found here -  DMR activity.

Possible NGSS topics to explore with students:

| Disciplinary Core Ideas: - HS-LS1-3 – Mechanisms of homeostasis - HS-LS2-1. - Use mathematical and/or computational representations - MS-ETS1-1 to 1.4 – Engineering design - MS-PS3-3 – Design that maximizes thermal energy transfer | Science & Engineering Practices: - Asking questions & defining problems - Evaluate competing designs - Constructing explanations & designing solutions | Crosscutting Concepts: - Systems & System Models - Cause & Effect - Energy & Matter |
| --- | --- | --- |

Background:

Light to Heat Each summer, there are horrifying stories of children and pets left in hot cars. Ultimately, most end up having a heat stroke and in many cases pass away. The inside of a car heats up so much faster than the outside due to the greenhouse effect. Rays of sunlight stream into the vehicle through the windows and strike the surfaces of the interior of the car. That visible light is absorbed and re-radiated as infrared light. Infrared radiation has a longer wavelength than visible light. The infrared radiation is unable to escape back through the windows. The trapped radiation causes the temperature inside the car to rise faster than the outside temperature. Stayin’ Cool! Mammals such as humans, dogs, and cats all have ways to regulate temperature, thermoregulation, to maintain homeostasis. Humans sweat to increase the removal of heat from the body through evaporation. Dogs usually pant to remove heat although they also have a small number of sweat glands in the pads of their paws. Cats will sprawl out on surfaces that are relatively cool to help remove body heat. They will lick their paws and rub the saliva on warmer parts of their bodies to increase evaporative cooling which is a similar mechanism as sweating in humans. When thermal regulation mechanisms are unable to maintain homeostasis, the mammal will go into heat distress. Heat distress leads to brain impairment, dehydration, heart failure, cell swelling, and protein denaturation… and potentially death. The problem with too much heat Students will conduct research into why mammals such as humans, dogs, and cats are unable to endure hot environments like closed cars in the summer for long periods. Then they design a solution using technology to protect car occupants from heat distress. Encourage them to refine their design until they have a working prototype. Have them compare their prototype with those of their classmates to determine which team has the “best” system.

(Final) Setup of the Project:

- Students may work in groups of two, three or four.

Supplies:

- TI-Innovator Hub and cable

- TI-84 Plus CE Python calculator

- 2x Grove Temperature sensor (inside and outside measurement)

- Grove Hall effect magnetic proximity sensor

- 2x Grove White LED Light

- Grove Continuous Servo motor

- 5 Grove Cables

- External USB Battery w/ micro USB Cable

- Fashion Doll Car, shoebox, or another object to ‘model’ a car

- Small toy pet

- Small magnet (ceramic, ferrite or ceramic-ferrite) for pet collar

- Small piece of clear plastic to model car window

- Cellophane tape to attach magnet to pet and window to motor

- Cling wrap (e.g. Saran) to enclose model car

- Safety scissors

The Challenges:

Challenge 1: Write a program that will play two sounds for 1 second each in a For loop that repeats five times.

Fig. 1: Import of complete Hub menu

Starting a New Program: Press [prgm] and then press 2: Python App to start a new Python program. Press Fns... and arrow over to Modul. Select 6: ti_hub ... , arrow over to ‘Advanced’ and press 1:from ti_hub import * (Fig. 1).

- from ti_hub import * brings in all of the functions on the Hub menus to be available for use in the program. This statement also establishes connection with the TI-Innovator Hub.

- Use a For loop to alternate between two different sound frequencies one octave apart.

Fig. 2: Fns Menu, for ...  loop

Loops are used to repeat a set of command. A For loop repeats a specified number of times. The programmer may define a For loop with four inputs:  a counter variable, a beginning value for the counter variable, a stop value for the counter variable and an optional step value variable.

- The for index in range( ) version of the For loop assumes that the index variable starts at 0 and increases by a step of 1 with each loop. If n is less than the stop value, the loop continues to repeat. The block of statements to repeat begins with a colon and includes the indented lines that follow.

- for index in range( ) is found under Fns...  Ctl  4: for i in range (size) (see Fig. 2).

Fig.3: Selecting the ‘Sound’ module

We now need to insert a sound command. Press Fns... and arrow over to Modul. Select 6: ti_hub ... ,  1: Hub Built-in devices,            and 3: Sound. This adds the sound module to your code (see Fig. 3).

- To add the type of sound output, press Fns... and arrow over to Modul again. The ‘Sound’ module is now available. Select 8: Sound ... and 1: tone (freq, time) as in Fig. 4.

- Following sound.tone with the sleep() function assures that the tone will complete before the program begins to play the next sound.

Sample program: PCA01.py

Fig. 4: Type of sound output

from ti_hub import *

from ti_system import *

import sound

for i in range(5):

sound.tone(440,1)

sleep(.5)

sound.tone(880,1)

sleep(.5)

Fig. 6: LEDs under Output devices

Fig. 5: Innovator Hub output devices

Challenge 2:  Use a For loop to blink two external LEDs. Try to blink 30 times with 1 second on and 1 second off for each blink.

- Start a New Program: Press [prgm] and then press 2: Python App to start a new Python program. Press Fns... and arrow over to Modul. Select 6: ti_hub ... , arrow over to ‘Advanced’ and press 1:from ti_hub import * (Fig. 1).

- Press Fns... and arrow over to Modul again. Select 6: ti_hub ... and select 3: Output devices ... 1: LED. (see Fig. 6)

- We defined our var as left_headlamp and right_headlamp (see Fig. 7)

- Use a For loop to make the LEDs turn off and on.

- Use sleep() to pause the program and create a blink effect. The number of seconds for the sleep() function determines how quickly the LEDs will blink.

Sample program: PCA02.py

Fig. 7: LEDs defined as headlights

from ti_hub import *

from led import *

from ti_system import *

left_headlamp=led("OUT 1")

right_headlamp=led("OUT 2")

for i in range(30):

left_headlamp.on()

right_headlamp.on()

sleep(1)

left_headlamp.off()

right_headlamp.off()

sleep(1)

Challenge 3: Connect a continuous servo motor to the TI-Innovator Hub and cause it to rotate clockwise (cw) and then in the opposite direction, counterclockwise (ccw).

Fig. 8: Hub Output devices

Fig. 9: Continuous servo from [Modul]

Start a new Python program as described above.

- Press Fns... and arrow over to Modul again. Select 6: ti_hub ... and select 3: Output devices ...                 6 : Continuous Servo (see Fig. 8).

- Press Fns... and arrow over to Modul again. Select 8: Continuous servo ... and 1: Continuous servo (“OUT 3”).

Fig. 10: Servo motor settings

Define window_motor as the variable name for continuous_servo.

- window_motor=continuous_servo(“OUT 3”) defines an external output device of the continuous servo motor type with the variable name window_motor. The statement associates the window_motor variable with the physical OUT 3 port of the Hub.

- Servo motors must be connected to the OUT 3 port because 5 volts are necessary to power the servo motor.

- Use .set_cw() (clockwise) and .set.ccw() (counter clockwise) functions to turn the motor in the desired direction.

- .set_cw() and .set_ccw() require two inputs, speed ( a value between 0 for off and 255 for maximum speed) and time in seconds for the motor to be on (see Fig. 10).

Sample program: PCA03.py

from ti_hub import *

from conservo import *

window_motor=continuous_servo("OUT 3")

window_motor.set_cw(20,1)

sleep(2)

window_motor.set_ccw(20,1)

sleep(2)

Challenge 4:  Connect a Grove temperature sensor to the TI-Innovator Hub and display the temperature reading on the calculator. Use a While loop to read and display temperature values until the [clear] key is pressed.

Fig. 11: Hub input devices

Fig. 12: Temp sensor in IN 1

- Start a new Python program, press Fns... and arrow over to Modul again. Select 6: ti_hub ... and select 2: input devices ...4 : Temperature (see Fig. 11).

- Press Fns... and arrow over to Modul again. Select 8: Temperature ... and 1: var=temperature (“IN 1”).

Fig. 13: Text display

The var temp_sensor_inside = temperature(“IN 1”) defines a sensor input of the temperature type with the variable name temp_sensor_inside. The statement associates temp_sensor_inside with the physical IN 1 port of the Hub.

- Use the .measurement() function of a temperature sensor to measure and return the current temperature reading.

- temp_inside=temperature_sensor_inside.measurement() stores the measurement value to a variable named temp_inside

- Add the disp_at command to display instructions for the user, in this case, “Press [clear] to quit.”

Fig. 14: While loop with ‘escape key’

Adding a while loop after the text display. While loops repeat the statements in the block if the condition at the top of the loop is true.

  - In the example, looping continues until the [clear] key is pressed (see Fig. 14).

  - disp_at( ) has options to build a message by joining together pieces of text with the + operator.

Sample program: PCA04.py

from ti_hub import *

from temperat import *

from ti_system import *

temp_sensor_inside=temperature("IN 1")

disp_at(5,"Press clear to quit","left")

while not escape():

temp_inside=temp_sensor_inside.measurement()

disp_at(5,"Temp inside Car = "+str(temp_inside)+" °C","left")

sleep (0.5)

Fig. 15: Hub Input device: Magnetic

Challenge 5:  Connect the Hall effect magnetic proximity sensor, which determines if a (relatively) strong magnetic field is close to the sensor. Display “Magnet is present” or “Magnet is not present” based on the reading of the Hall effect sensor and the position of the magnet.

- Again, start a new Python program, press Fns... and arrow over to Modul again. Select 6: ti_hub ... and select 2: input devices ...6 : Magnetic (see Fig. 15).

- The Hall effect sensor detects the presence (or absence) of a magnetic field with south polarity.

- Press Fns... and arrow over to Modul again. Select 8 : Magnetic... and 1: var=magnetic (“port”).

- magnet_sensor defines the variable we are measuring and stores the measurement value to a variable named magnet_value.

Fig. 16: Defining/storing the variable

It is recommended that IN 3 is used to supply enough power to the sensor.

- Use the if..else control structure with boolean comparative statements to determine if a magnet is close or not.

- Sensor values are related to voltage measurements. When the south pole of the magnet is close, readings are usually under 200.

- Use disp_at to indicate whether or not a magnet is present (Fig. 17).

Fig. 17: Defining/storing the variable

Sample program: PCA05.py

from ti_hub import *

from ti_system import *

from magnetic import *

magnet_sensor=magnetic( "IN 3" )

disp_at(5,"Press clear to quit","left")

while not escape():

magnet_value=magnet_sensor.measurement()

disp_at(3,"magnetic sensor (bits)= " +str(magnet_value),"left" )

if magnet_value<200:

disp_at(5,"magnet is present ","left")

else:

disp_at(5,"magnet is not present ","left")

sleep(.5)

Fig. 18: Importing required modules

Final Challenge – Putting it all together:   Develop a car alarm system that determines if a pet is present (magnet) and if the temperature inside the car is above a critical threshold before triggering the alarm. This challenge has students create a system that reads data from inputs such as temperature and the Hall effect magnet sensor which in turn controls LEDs, sounds through the Hub’s speaker, and the continuous servo motor.

- Make sure students import all the modules required for the different parts of the final code (see Fig. 17).

- For each of the sensors and output devices, variables need to be defined and stored for display (see Fig. 19).

- The critical temperature and Hall sensor reading will, depending on their values, trigger a reaction of the output devices (see Fig. 20).

Fig. 20: Setting the alarm thresholds

Fig. 19: Defining/storing the variables

In the sample code below, lines with # are used to explain functions or give specific instructions.

- They can be omitted in the final code to reduce the number of lines/size of the program.

Sample program: PCAFINAL.py

from ti_hub import *

from ti_system import *

import sound

from led import *

from magnetic import *

from temperat import *

from conservo import *

# Define sensor inputs

temp_sensor_inside=temperature("IN 1")

temp_sensor_outside=temperature("IN 2”)

magnet_sensor=magnetic("IN 3")

# Define alarm response outputs

left_headlamp=led("OUT 1")

right_headlamp=led("OUT 2")

window_motor=continuous_servo("OUT 3")

# Display system information

disp_at(1,"Pet Car Alarm is on ", "left")

disp_at(10,"Press clear to turn off ","left")

# Set initial value for window as closed

window= "closed"

# Set up main while loop for the alarm system

while not escape():

# Read sensors and store values to variables

temp_inside=temp_sensor_inside.measurement()

temp_outside=temp_sensor_outside.measurement()

magnet_value=magnet_sensor.measurement()

# Report status of alarm system

# Window closed or open, pet in car,

# temperature values

if magnet_value<200:

disp_at(5,"Pet in car  ","left")

else:

disp_at(5,"Pet not in car" ,"left")

if window=="closed":

disp_at(6,"Window is closed" ,"left")

else:

disp_at(6, "Window is open  " ,"left")

disp_at(7, "Temperature inside= "+str(temp_inside), "left")

disp_at(8, "Temperature outside= "+str(temp_outside), "left")

# Detect an alarm situation

if temp_inside>25 and magnet_value<200:

# Alarm detected response

disp_at(3, "Warning! Pet is in heat distress" ,"left")

left_headlamp.blink(5,3)

right_headlamp.blink(5,3)

sound.tone(440,.5)

sleep(.5)

sound.tone(880,.5)

sleep(.5)

# Open window, if closed

if window== "closed":

window_motor.set_cw(30,.25)

sleep(.25)

window= "open"

# If no problem detected,

# set alarm outputs off

else:

disp_at(3, "No problems detected ","left")

left_headlamp.off()

right_headlamp.off()

#  If open, close window

if window=="open":

window_motor.set_ccw(30,.25)

sleep(.25)

window= "closed"

sleep(.5)

# When [clear] is pressed, close down system

disp_at(1, "Pet Car Alarm is off","left")

left_headlamp.off()

right_headlamp.off()