Overview:

This project assumes working knowledge of the programming concepts introduced in the Digital Mood Ring project. Please refer to that project as a review of concepts. In this smart irrigation project, you will build and program a smart irrigation system with a feedback and control loop. The system uses three input modules: a temperature-humidity sensor, a soil moisture sensor, and an ambient light level sensor. The system has a single output to a power control unit with a water pump connected. The final program is written to read all four input parameters and logically compare them with critical values and among each other to determine when to turn the water pump on and off. The project is presented in a series of small challenges that build the knowledge and skills required for the final open-ended challenge. The final challenge also relies on your understanding of some important biology and ecology topics that are relevant to optimizing the system you ultimately design and refine.

Possible NGSS topics to explore with students:

| Disciplinary Core Ideas - MS-LS 2-3, LS 2-5 -- Ecosystems Interactions - MS-ESS 2-4 – Water Cycle - MS-ETS 1-2, MS-ETS 1-4 -- Design and evaluate a solution to a problem - HS-LS 2-1. Use mathematical and/or computational representations | Crosscutting Concepts - Stability & Change - Cause & Effect | Science and Engineering Practices - Constructing explanations & designing solutions - Evaluate competing designs - Asking questions & defining problems |
| --- | --- | --- |
| Background: |  |  |  |  |

Identify the problem

Humans invented agriculture over 6,000 years ago to produce more food than could be hunted and gathered. This increase in food availability produced a rapid increase in population that was totally dependent on agriculture for survival. Today, the world’s large population requires sophisticated large-scale agriculture to keep everyone fed. Climate changes can have severe consequences on the food production required to keep the large human populations fed. Natural and man-made ecological disasters, such as the Dust Bowl of the 1930s, can have severe consequences on the dependent populations. Science and technology can help to optimize food production and mitigate the effects of climate change and poor farming practices.

Build a Solution

In this project, you are challenged to use science and technology to design and build a system that utilizes a limited amount of water in a “smart” way to grow crops. To achieve this, you need some STEM skills and knowledge from the disciplines of science, electronics, logic and computer programming. To enable you to complete this project, you are challenged to first complete smaller tasks that will provide the programming and electronics knowledge required.

Some of the following science principles will be worked into the activity:

- The effect of air temperature on the evaporation rate of water? Should water be used when it is hot or cool?

- How does soil type affect the percolation of water into the ground? Should water be delivered in a strong rush? Or should it be delivered slowly and in pulses?

- Does relative humidity affect the evaporation rate of water? Does warm air hold more water than cool air? What is relative humidity? Should water be delivered when relative humidity is high or low?

- How does the soil moisture level affect the rate that water can be absorbed? At what soil moisture level should water be delivered to crops?

- Does photosynthetic activity affect the uptake of water? Should plants be watered during daylight or night?

Supplies:

- Possible supplies for building garden model:

  - Drinking straws

  - Duct Tape (always useful)

  - Container for the plants, such as a 1-gallon milk jug

  - Soil, perlite or some other growth medium

- Possible supplies for building garden model:

  - Drinking straws

  - Duct Tape (always useful)

  - Container for the plants, such as a 1-gallon milk jug

  - Soil, perlite or some other growth medium

- TI-Innovator Hub and cable

- TI Nspire CXII calculator

- Grove – Temperature & Humidity Sensor

- Grove – Soil Moisture Sensor

- Grove – Light Sensor

- Grove – MOSFET for power control module with        4 x AA battery holder

- 4 x AA batteries to provide power to the pump (required)

- Water Pump with the plastic tube

- TI-Innovator Hub and cable

- TI Nspire CXII calculator

- Grove – Temperature & Humidity Sensor

- Grove – Soil Moisture Sensor

- Grove – Light Sensor

- Grove – MOSFET for power control module with        4 x AA battery holder

- 4 x AA batteries to provide power to the pump (required)

- Water Pump with the plastic tube

Setup Project:

Fig. 1: Sensors and pump control connected to Innovator Hub

Fig. 1: Sensors and pump control connected to Innovator Hub

- Your teacher may assign you to work in groups of two, three or four.

- Grove Soil Moisture Sensor (IN 3), Temperature & Humidity Sensor (IN 2), and Light Sensor (IN 1) plugged into the IN ports of the Innovator Hub

- Grove – MOSFET for power control module with   battery holder (OUT 1)

Challenge 1:  Write a program named WATERC1 that continuously measures and displays the ambient light level. The program should:

- Connect a light level sensor to IN 1.

- Range the light level reading from 0 to 100.

- Use a while .... loop to continuously read and display light level every ½ second on the calculator screen.

- Enable the [clear] key to quit the while loop and end the program.

Guidance for Challenge 1:

- The light level sensor is a device that has a large range of responses to light intensity. As a result, the output is not linear with light intensity. In addition, the device does not measure with a particular unit. Instead, the device returns a raw value from 0 to 214. It is useful to use the RANGE command to scale the output from 0 to 214 to 0 to 100.

- Use the Python functions descriptions described above to review the use of a while loop. The ‘while not escape’ loop can be found in the ti_system module. On the TI-84 Plus CE the [clear] key acts as an ‘escape’ key. Note that all the statements inside the ‘while not escape’ loop are indented.

- Use the Python function descriptions above to review the use of the disp_at() function, also found in the ti_system module for displaying values on the screen.

Fig. 2: Hub and system modules

Fig. 2: Hub and system modules

Reminder: The input port you choose in your code (in this case IN 1) must match where the sensor is actually plugged in. Sample code can be found in Fig. 2.

Challenge 2: Write a program named WATERC2 that measures soil moisture every two seconds for a total of twenty times. The program should display if the soil is dry or moist based on the sensor reading.

Guidance for Challenge 2: - In Challenge 1, all of the necessary steps of the program were given to the students. In Challenge 2, help the students to analyze the task and make a list of the necessary steps to accomplish the task. - Discuss the differences in the control structure between Challenge 1 and Challenge 2. The While loop provides indefinite program execution and the For loop provides definite program execution. - The soil moisture sensor produces an output from 0 to 214 based on the conductivity (dielectric permittivity) of the soil which is dependent on water content. Like the light level sensor, it is useful to use the RANGE function to scale the sensor output from 0 to 100. - The sensor could be attached to any of the input ports 1, 2 or 3. It is essential that the port used in the statement that defines the sensor object matches the port that the physical sensor is plugged into. - Discuss how the different types of soils like clay, sand, and loam and water interacts with each. Features to consider are the rate of absorption, water capacity, compaction, and runoff. Clay; Sand; Loam. Soaks in water slowly; Soaks in water very quickly; Soaks moderately. Holds water very well; Holds water poorly; Holds water moderately. Fine grain texture; Coarse grain texture; Mixed grain texture. Severe erosion; Low erosion; Low erosion. - Discuss the best conditions for plant soil watering. Should the soil be allowed to dry out between watering? Do different plants require different condition? - As an extension, you may determine values to indicate the relative ranking of soil moistures. To do this: - Place soil into three different pots labeled: Dry, moist and soaked. - Add water to each pot to the labeled soil moisture. - Consistently insert the soil moisture sensor tines (prongs) halfway into the soil of the pot labeled “dry” and run the sensor program and record the displayed soil moisture value. - Repeat for the other two soil moisture levels. - Use the three measured values in your program to rank the soil moisture of your plants and decide when to water the plant. Fig. 3: Functions of Moisture Sensor Fig. 3: Functions of Moisture Sensor - The moisture module can be found under the Hub module, 6: ti_hub, 2: Input devices … - Once you have imported the module, it will be available with the functions shown in Fig. 3. The var name is moisture, which is then measured in 2: var.measurement (). Fig. 5: If .. Else... loop for conditional statements Fig. 5: If .. Else... loop for conditional statements Fig. 4: For loop for repeat measurements Fig. 4: For loop for repeat measurements Use a for loop that will read and display twenty moisture measurements. In the program editor, press Fns ... , arrow over to Ctl and select 4: for i in range (size). (See Fig. 4). - Next, we need to add a “decision statement” (if....else....loop). Again, press Fns ... , arrow over to Ctl and select 2: if ... else ... (see Fig. 5).

Challenge 3:   Write a program named WATERC3 that connects the Digital Humidity and Temperature (DHT) sensor.

- Read 20 measurements at two-second intervals and display with an appropriate message.

- Use a decision tree based on the temperature and relative humidity measurements to determine the present watering conditions as indicated in the graph on the next page (Fig. 6).

- Display an appropriate message with each of the four cases.

Guidance for Challenge 3:

- The amount of water from the ground that evaporates into the air is dependent on the ambient temperature and relative humidity.

- For a particular temperature, there is a maximum amount of water that can evaporate and exist as vapor in the air.

- Relative humidity is the percentage of the measured vapor pressure divided by the theoretical saturation vapor pressure. Thus, relative humidity does not inform how much water is in the air, instead, the percentage of the theoretical maximum amount of water that is in the air at a particular temperature.

- When relative humidity is 100%, and water evaporates from the ground, water vapor will precipitate back into the liquid phase and fall as rain and there will be no net transfer of liquid water from the ground into the air.

- There are many inferences that can be made from the above concept.

  - On a hot day, more water will evaporate from the ground to produce a relative humidity of 70% compared to a cold day with 70% relative humidity. This is because on a hot day there is a greater saturation vapor pressure and more water in the air is needed to establish 70%.

  - When relative humidity is high, less evaporated water is red to reach the maximum saturated vapor density at a given temperature.

  - To minimize the net transfer of liquid water on the ground into water vapor in the air (evaporation), water should be delivered when it is cool and the relative humidity is high.

Fig. 6: Relative Humidity vs. Temperature Chart

Fig. 6: Relative Humidity vs. Temperature Chart

Look at the graph on the right and determine the conditions for each of the four regions. Use these conditions to construct an if ... elif ... else decision tree in your program.

- The DHT sensor requires a few seconds to “warm up” and begin communicating with the Hub. During this period, the temperature may be reported to be absolute zero (-273 °C). The program uses a While loop to continuously read the DHT every one second until it reports a value greater than absolute zero.

- Add all the requited modules to your new program, WATERC3.

- The DHT module can be found under the Hub module, 6: ti_hub, 2: Input devices … 1: DHT- Digital Humidity & Temp (Fig. 7).

- We chose htsensor as the name for the var. in =dht (“port”, “opt”), as shown in Fig.9.

Fig. 9: Modules and sensor selection

Fig. 9: Modules and sensor selection

Fig. 8: Functions of the DHT sensor

Fig. 8: Functions of the DHT sensor

Fig. 10: Sample if..elif..else..loop

Fig. 10: Sample if..elif..else..loop

Fig. 7: DHT input device for Hub

Fig. 7: DHT input device for Hub

- From the ti_system module, we need to add a “decision statement,” (if...elif...else...loop). Again, press Fns ... , arrow over to Ctl and select 3: if ... elif ... else ...

- The values you select for your ‘decision’ statements may depend on the conditions in your classroom/lab.

- Parts of a sample program for WATERC3 are shown in Fig. 10 on the right.

Challenge 4: Write a program named WATERC4 using a While loop to continuously measure and display a dashboard of all sensor value readings. The user should be able to stop the monitoring by pressing the [clear] key

Guidance for Challenge 4:

Fig. 11: Modules for Hub and input Sensors

Fig. 11: Modules for Hub and input Sensors

- The program uses most of the code from the previous three challenges.

- Make sure that all the required modules are imported (see Fig. 11)

- All of the sensors need to be connected as they were in the previous challenges.

- Use the while loop with the [clear] key as an exit.

- This program can later be modified for the final challenge by adding the next challenge and control logic.

Fig. 12: Sample Display of Measurements

Fig. 12: Sample Display of Measurements

- Use the disp_at function (ti_system module) to show the readings of all three sensors at different lines (see Fig. 12).

Challenge 5: Write a program named WATERC5 to connect the pump power module and run the pump for 10 seconds. Be sure to turn the pump off at the end of the program. Try setting the pump power to different values.

As an extension, try to estimate the pump flow rate in mL/sec.

Guidance for Challenge 5:

- The Innovator Hub can control the amount of power delivered to the submersible pump using an attached power module with a battery pack.

Fig. 13: Illustration of the transistor-based Power control

Fig. 13: Illustration of the transistor-based Power control

The power module has a special type of transistor called a MOSFET mounted on the unit. Fig. 13 shows the analogy between electricity flowing in a circuit and water flow through a system to help students to better understand how a transistor works in an electrical circuit.

- When the Python set output value function for an analog_out object is executed (e.g. pump.set(255)), the Hub creates a control signal on the OUT port. The execution of the Python set analog_out function is similar to the brain sending a command to the hand in step 1 of the analogy.

- When a power module is connected to one of the Hub’s OUT port, that control signal is sent to the transistor. This signal causes the transistor to adjust the amount of electricity that will flow through it. In step 2 of our water analogy, this signal can be thought of as the hand turning on the faucet.

- When the transistor is turned on, more current flows through it and into the motor. Like in step 3 of the analogy, when the faucet is opened, more water flows onto the water wheel.

- When more electrical current flows through the motor, greater power is delivered to the shaft causing it to turn faster. As in step 4 of the analogy, more water flowing onto the paddles of the water wheel cause it to turn faster.

- In this challenge, the power module is used to control the pump. A pump is just a motor with an impeller and housing attached to its shaft.

Fig. 15: Pump analog setup, part 2

Fig. 15: Pump analog setup, part 2

Fig. 14: Pump analog setup

Fig. 14: Pump analog setup

Caution: Be sure to SET the power module off (e.g. pump.set(0) or  pump.off() for an analog_out object named pump.) before exiting the program. If your program fails to turn off the module, the pump will continue to run even after the program has quit.

- The pump is added as an analog_out device. It can be found under the Hub module, 6: ti_hub, 3: Output devices … 7: Analog out.

- Once the Analog out module is added to the list, select 1: var=analog_out (“port”). The pump will be connected to the “OUT 1” port.

- As an extension, we can calculate the flow rate of the pump. To do this, we will run the pump tubing into an empty graduated cylinder and fill the cylinder for a measured amount of time. Use the water volume and run time to calculate the flow rate.

- How does changing the power value setting of the pump changes the flow rate?

- Write a program (we called ours PUMPCAL) to determine the flow rate for the pump at different settings.

Fig. 16: Water volume und Pump settings

Fig. 16: Water volume und Pump settings

Fig. 17: Volume vs. Pump Setting

Fig. 17: Volume vs. Pump Setting

In our example, we ran the pump for ten seconds or less at 10 different pump settings [L1] and recorded the amount of water in mL/sec in [L4].

- Fig. 23 shows the amount of water in mL/sec as it is related to the pump setting  (0 – 255).

Fig. 18: Sample program PUMPCAL

Fig. 18: Sample program PUMPCAL

- The for .... in range()  loop allows you to run the pump at 10 different pump settings for ten seconds each time. The sleep() command can be modified if too much water is pumped during that time.

- Fig. 18 shows a sample program for the extension:  PUMPCAL

Final Challenge:   Write a program that uses all of the sensors from the previous challenges. Continuously monitor light level, soil moisture, air temperature, and humidity and display the current value on the calculator. Use your knowledge of ecology, biology, and earth science to determine the best conditions to water your garden. When the conditions are favorable, set the pump to deliver water at the best rate for your garden. Test your system with these conditions:

- light level < 20

- soil moisture < 10

- temperature < 25

- humidity > 80

When these conditions are true, set the pump to 255 (max. output). When the conditions are false, set the pump to zero.

Guidance during the Final Challenge:

- Think about the concept of optimization and how that is expressed in the logic of a program.

- For each sensor, consider the conditions suited to watering. These values are called sensor set-points. For example, a set-point for temperature could be 25°C and watering should only occur when the measured temperature is less than that temperature set-point.

| Sensor | Sensor Set-Point | Set-point condition | Conditional expression |
| --- | --- | --- | --- |
| Light Level | 20 | < | light<20 |
| Air Temperature | 25 | < | temperature<25 |
| Air Humidity | 80 | > | humidity>80 |
| Soil Moisture | 10 | < | moisture<10 |

- Once the four conditional expressions are determined in the table above, use the logical operators AND and OR to link those expressions together to be evaluated by a single if ... then ... else conditional statement.

- When AND is used, both conditional expressions must be TRUE for the statement to be TRUE.

- For example, to construct a statement that requires the temperature to be less than the set-point of 25°C and the humidity to be greater the set-point of 80% to water, use:

if temperature<25 and humidity>80:

pump.set(255)

else:

pump.set(0)

- The statement above ensures that the pump will turn on at maximum flow rate only when the temperature is cool

and the humidity is high.

- When OR is used, either or both conditional expressions must be TRUE for the OR statement to be TRUE.

- For example, to construct a statement that requires the light to be less than the set-point of 20 or the moisture to be less than the set-point of 10 to water.

if light<20  or  moisture<10:

pump.set(255)

else:

pump.set(0)

- The pump will turn on when either it is dark or the soil is dry. Note, the pump will also turn on when it is dark and the moisture is low.

- Since it can be difficult to consider four conditional expressions concurrently, it may be best to begin with only one condition and then add an additional condition with a logical operation. Once successful with two conditional expressions, you can then add additional conditions until all four are utilized.

- The pump control statement in the example below does the following:

  - 1) runs the pump under any environmental conditions if the soil is very dry, soil moisture measurement less than 10, or

  - 2) runs the pump when the environmental conditions are good for watering if the soil is moderately dry, soil moisture measurement less than 60.

- The pump will not run under any environmental conditions if the soil is wet.

- Note: Experiment by taking soil moisture measurements with your soil to determine whether 10 and 60 are good values for your specific environment.

if light<20 and temperature<25 and humidity>80 and moisture<60 or moisture<10:

disp_at(7,"good time to water","left")

pump.set(255)

else:

disp_at(7,"bad time to water","left")

pump.set(0)

Fig. 19a: Copying a previous program

Fig. 19a: Copying a previous program

You can save time by building the final program from a copy of the WATERC4.py program.

- Press prgm and 2:Python App; select the program and press Manage , then 1: Replicate Program ... and rename the existing program (we called ours “WATERFIN”). See Fig. 19a and 19b for details.

Fig. 19b: Copying a previous program

- The copy of the WATERC4 program sets up sensors and a while... loop to take measurements.

- Next, you need to define the pump analog_out object in the sensor set up section at the top of the program similar to what you did in WATERC5.

Fig. 20: Sample code for decision loop

Fig. 20: Sample code for decision loop

And finally, add if .. then .. else logic and pump statements within the loop.

- Set up if .. then .. else logic to run the pump when conditions are good and soil is not wet or when the soil is dry.

- Remember that, if your program fails to turn off the module, the pump will continue to run even after the program has quit.